Nortel Communication Server 1000M, Release 4.0 to Cisco IOS Voice Gateway using T1 QSIG with H.323

#### October 30, 2007 Revision 4

Table of Contents

Introduction	1

Network Topology	2

Hardware Requirements	2

Software Requirements	2

Features	3

Features Supported	3

Limitations	3

Configuration	4

Configuring the Nortel CS1000M:  Switch 1 PBX ‘A’ (ENBLOC)	4

Configuring the Nortel CS1000M:  Switch 2 PBX ‘B’	18

Configuring the Cisco 2801	31

Configuring the Cisco 3845	38

Acronyms	45

# Introduction

- Although specific gateway router models were used to validate its content, this application note also applies to all Cisco 1700/2600/3600/3700/2800/3800 series Cisco IOS voice gateways.

- This application note provides configuration guidelines for a toll-bypass network using Cisco IOS voice gateways to connect Nortel Communication Server 1000M, Release 4.0 PBXs.  The PBXs are connected to the Cisco IOS voice gateways by T1 QSIG trunk circuits.  The Cisco IOS voice gateways “extend” the T1 QSIG trunk circuits with VoIP, using the H.323 protocol.

- Two Nortel Communication Server 1000M, Release 4.0 PBXs were connected via T1 QSIG trunk to two Cisco IOS voice gateways.  The voice gateways were connected via IP over Ethernet, and configured for VoIP using H.323.   End-to-end calls were placed between the PBXs to exercise and test basic calls as well as QSIG supplementary services such as MWI, call hold, call transfer, call conference, and call forward.

- Using the Nortel PBX configurations and Cisco IOS voice gateway configurations in this application note, successful toll bypass integration was achieved.  This includes basic call, call transfer, call conference, and call forward, with some limitations on Caller ID features during transfer, forward, hold and conference scenarios.   These limitations are detailed in the following sections and most were found to be inherent to the Nortel PBXs.

# Network Topology

- Network Topology or Test Setup

## Hardware Requirements

- (2) Cisco IOS voice gateways with T1 VWICs (voice/WAN interface cards)

- (2) Nortel Communication Server 1000M PBXs

- (1) Nortel Call Pilot Voice Mail System

- (4) Nortel digital station telephones

## Software Requirements

- Nortel CS1000M Release 4.0

- Cisco IOS voice gateways:  Cisco IOS Release Version 12.4(2.10) or later.

# Features

## Features Supported

- Basic Call (ENBLOC and Overlap)

- Call Transfer: Supervised Local Transfer

- Call Transfer: Supervised Network/External Transfer

- Call Conference: Local

- Call Conference: Network/External

- Call Forward: Local

- Call Forward: Network/External

- Call Hold

- MWI

# Limitations

- For basic calls, both overlap and ENBLOC, the called number is not passed to the originating phone in the ALERTING message.  Initially the dialed number (prior to CONNECT) is displayed. After CONNECT, the connected number is displayed on the originating phone. This is inherent to the PBXs and also occurs with the PBXs connected directly via a T1 QSIG trunk.

- During call transfers, the original calling name and number were displayed on the final destination phone only after the transfer was complete. This is inherent to the PBXs and also occurs with the PBXs connected directly via a T1 QSIG trunk.

- For conference calls, connected name and number were not updated on the originating phone when a conferee dropped out. This is inherent to the PBXs and also occurs with the PBXs connected directly via a T1 QSIG trunk.

- For conference calls, the calling name and number were generally not passed to the remaining conferee when the conferencing phone dropped out.   The only exceptions were conference calls accomplished by an external call followed by a local conference from the original called party (e.g., phone A calls phone C, then phone C conferences in phone D).  In the rest of the cases, either the conferencing phone’s name / number is displayed, or the trunk number is displayed. This is inherent to the PBXs and also occurs with the PBXs connected directly via a T1 QSIG trunk.

- It is important to set EXTT parameter to “yes” under LD 15 in the PBX configuration to avoid dropped calls on certain conference scenarios when the initiating/conferencing party drops (e.g., Phone A calls Phone C, Phone A conferences in Phone D, and Phone A drops).

- For forwarded calls, the original calling name was generally not displayed on the final destination.  Only the forwarding name was displayed.  This is inherent to the PBXs and also occurs with the PBXs connected directly via a T1 QSIG trunk.

- For forwarded calls, the final destination number was updated at the original side before it connects.  This is inherent to the PBXs and also occurs with the PBXs connected directly via a T1 QSIG trunk.

- For forwarded calls accomplished by a local call followed by an external forward (e.g., phone A calls phone B and phone B forwards to phone C), there was a long (~5sec) pause before the call was connected.

- For a call hold on a basic call, when the originating phone places the destination phone on hold, the final destination number disappears when hold is invoked and does not reappear when it is released.  This does not occur if the destination phone places the originating phone on hold.  This is inherent to the PBXs and also occurs with the PBXs connected directly via a T1 QSIG trunk.

# Configuration

## Configuring the Nortel CS1000M:  Switch 1 PBX ‘A’ (ENBLOC)

Note:  It is important to set EXTT parameter to “yes” under LD 15 to avoid dropped calls on certain conference scenarios when the initiating/conferencing party drops (e.g., Phone A calls Phone C, Phone A conferences in Phone D, and Phone A drops).

********************************************************************************

COMMON EQUIPMENT CONFIGURATION

********************************************************************************

ld 22

REQ  prt

TYPE cequ

CEQU

MPED 8D

SUPL  000  004  008  012

016  032  036  040

044  048  064  068

072 V096 V100

TDS   000

CONF  029  030  031  062

094  095

DLOP  NUM DCH FRM TMDI LCMT YALM T1TE TRSH

PRI  02   24  ESF NO   B8S  FDL  -    00

06   23  ESF NO   B8S  FDL  -    00 ...... (T1 PRI card/slot used)

07   23  ESF NO   B8S  FDL  -    00

PRI2 04 05

DTI2

MISP

REQ

********************************************************************************

DIGITAL DATA BLOCK CONFIGURATION

********************************************************************************

>ld 73

DDB000

MEM AVAIL: (U/P):p 2820901    USED U P: 206457 69217    TOT: 3096575

DISK RECS AVAIL: 1152

REQ  prt

TYPE ddb

CC0  6

PREF CC0  6

SREF CC0  FREE RUN

TRSH 00

RALM 128

BIPC 0

LFAC 0

BIPV 4 4

SRTK 24 3600

SRNT 1024 1024

LFAL 10240 10240

SRIM 1

SRMM 2

ICS

REQ

********************************************************************************

D-CHANNEL CONFIGURATION

********************************************************************************

>ld 22

PT2000

REQ  prt

TYPE adan dch 6

ADAN     DCH 6

CTYP MSDL

CARD 06

PORT 1

DES

USR  PRI

DCHL 6

OTBF 32

PARM RS422  DTE

DRAT 64KC

CLOK EXT

IFC  ISGF ............... (For ISO Q>SIG configuration with GF Platform)

PINX_CUST 0

ISDN_MCNT 300

CLID OPT0

CO_TYPE  STD

SIDE NET

CNEG 1

RLS  ID  **

RCAP COLP NDI CCBI CCNI PRI DV3I CTI  QMWI

PR_TRIGS DIV 2 3

CNG 2 3

CON 2 3

CTR1 2 3

PR_RTN NO

MBGA NO

OVLR NO

OVLS NO

T310 120

T200 3

T203 10

N200 3

N201 260

K    7

********************************************************************************

ROUTE DATA BLOCK CONFIGURATION

********************************************************************************

>ld 21

PT1000

REQ: prt

TYPE: rdb

CUST 0

ROUT 106

TYPE RDB

CUST 00

DMOD

ROUT 106

DES  CS01

TKTP TIE

NPID_TBL_NUM   0

ESN  NO

CNVT NO

SAT  NO

RCLS EXT

VTRK NO

NODE

DTRK YES

BRIP NO

DGTP PRI

ISDN YES

MODE PRA

IFC  ISGF

SBN  NO

PNI  00000

NCNA NO

NCRD NO

CHTY BCH

CTYP UKWN

INAC NO

ISAR NO

CPFXS YES

DAPC NO

INTC NO

DSEL VOD

PTYP PRI

AUTO NO

DNIS NO

DCDR NO

ICOG IAO

SRCH RRB

TRMB YES

STEP

ACOD 206

TCPP NO

TARG 01

CLEN 1

BILN NO

OABS

INST

ANTK

SIGO STD

ICIS YES

TIMR ICF  512

OGF  512

EOD  13952

NRD  10112

DDL  70

ODT  4096

RGV  640

GRD  896

SFB  3

NBS  2048

NBL  4096

IENB  5

TFD  0

VSS  0

VGD  6

DRNG NO

CDR  NO

VRAT NO

MUS  NO

FRL  0 0

FRL  1 0

FRL  2 0

FRL  3 0

FRL  4 0

FRL  5 0

FRL  6 0

FRL  7 0

OHQ  NO

OHQT 00

CBQ  NO

AUTH NO

TTBL 0

ATAN NO

PLEV 2

ALRM NO

ART  0

SGRP 0

AACR NO

********************************************************************************

TRUNK DATA BLOCK CONFIGURATION

********************************************************************************

>ld 20

PT0000

REQ: prt

TYPE: tnb

TN   6 1

DATE

PAGE

DES

DES

TN   006 01

TYPE TIE

CDEN SD

CUST 0

TRK  PRI

PDCA 1

PCML MU

NCOS 0

RTMB 106 1

B-CHANNEL SIGNALING

TGAR 1

AST  NO

IAPG 0

CLS  UNR DTN WTA LPR APN THFD HKD

P10 VNL

TKID

AACR NO

DATE 18 MAY 2005

********************************************************************************

COORDINATED DIALING PLAN CONFIGURATION (TO DIAL 53xx)

********************************************************************************

>ld 87

REQ  prt

CUST 0

FEAT cdp

TYPE dsc

DSC  53

DSC  53

FLEN 0

DSP  LSC

RLI  6

NPA

NXX

MEM AVAIL: (U/P): 2820901    USED U P: 206457 69217    TOT: 3096575

DISK RECS AVAIL: 1152

REQ

9

********************************************************************************

ROUTE LIST BLOCK CONFIGURATION

********************************************************************************

>ld 86

ESN000

MEM AVAIL: (U/P): 2820901    USED U P: 206457 69217    TOT: 3096575

DISK RECS AVAIL: 1152

REQ  prt

CUST 0

FEAT rlb

RLI  6

9

RLI  6

ENTR 0

LTER NO

R

9OUT 106

TOD  0 ON  1 ON  2 ON  3 ON

4 ON  5 ON  6 ON  7 ON

VNS  NO

CNV  NO

EXP  NO

FRL  0

DMI  0

FCI  0

FSNI 0

SBOC NRR

IDBB DBD

IOHQ NO

OHQ  NO

CBQ  NO

ISET 0

NALT 5

MFRL 0

OVLL 2

MEM AVAIL: (U/P): 2820901    USED U P: 206457 69217    TOT: 3096575

DISK RECS AVAIL: 1152

REQ  ****

OVL000

********************************************************************************

DIGITAL STATION CONFIGURATION

********************************************************************************

>ld 11

SL1000

MEM AVAIL: (U/P): 2820901    USED U P: 206457 69217    TOT: 3096575

DISK RECS AVAIL: 1152

DIGITAL TELEPHONES AVAIL:     4    USED:     4    TOT:     8

IP USERS AVAIL:     4    USED:     4    TOT:     8

BASIC IP USERS AVAIL:     7    USED:     1    TOT:     8

ACD AGENTS AVAIL:    10    USED:     0    TOT:    10

PCA AVAIL:     0    USED:     0    TOT:     0

AST     AVAIL:     1    USED:     0    TOT:     1

TNS     AVAIL:  2302    USED:   198    TOT:  2500

DATA PORTS AVAIL:  2500    USpED:     0    TOT:  2500

REQ: prt

TYPE: 2616

TN   001 0 0 0

DATE

PAGE

DES

DES  CS101A

TN   001 0 00 00

TYPE 2616

CDEN 8D

CUST 0

AOM  0

FDN  2500

TGAR 1

LDN  NO

NCOS 0

SGRP 0

RNPG 0

SCI  0

SSU

XLST

CLS  CTD FBA WTA LPR MTD FNA HTA ADD HFD

MWA LMPN RMMD SMWD AAD IMD XHD IRD NID OLD VCE DRG1

POD DSX VMD CMSD SLKD CCSD SWD LND CNDA

CFTA SFD MRD DDV CNID CDCA MSID DAPA BFED RCBD

ICDD CDMD LLCN MCTD CLBD AUTU

GPUD DPUD DNDA CFXA ARHD CLTD ASCD

CPFA CPTA ABDD CFHD FICD NAID BUZZ AGRD MOAD AHD

DDGA NAMA

DRDD EXR0

USRD ULAD RTDD RBDD RBHD PGND OCBD FLXD FTTC DNDY DNO3 MCBN CDMR

CPND_LANG ENG

RCO  0

EFD  2500

HUNT 2500

EHT  2500

LHK  0

PLEV 02

CSDN

AST

IAPG 0

AACS NO

ITNA NO

DGRP

MLWU_LANG 0

DNDR 0

KEY  00 SCR 2308 0     MARP

CPND

NAME ZEUS8

XPLN 5

DISPLAY_FMT FIRST,LAST

01

02

03 CFW 4  2309

04 AO6

05 TRN

06

07

08

09

10

11

12

13

14

15 RGA

DATE  6 JUL 2005

NACT ****

12

OVL000

********************************************************************************

PBX SOFTWARE RELEASE

********************************************************************************

ld 22

REQ  iss

CALL SERVER/MAIN CAB

VERSION 2121

RELEASE 4

ISSUE 00 T  +

IDLE_SET_DISPLAY NORTEL

********************************************************************************

PACKAGES INSTALLED

********************************************************************************

REQ  prt

TYPE pkg

OPTF                  1

CUST                  2

CDR                   4

CTY                   5

RAN                   7

TAD                   8

DNDI                  9

EES                   10

INTR                  11

ANI                   12

ANIR                  13

BRTE                  14

DNDG                  16

MSB                   17

SS25                  18

DDSP                  19

ODAS                  20

DI                    21

CHG                   23

CAB                   24

BAUT                  25

CASM                  26

CASR                  27

BQUE                  28

NTRF                  29

NCOS                  32

CPRK                  33

SSC                   34

IMS                   35

UST                   35

UMG                   35

ROA                   36

NSIG                  37

MCBQ                  38

NSC                   39

BACD                  40

ACDB                  41

ACDC                  42

LMAN                  43

MUS                   44

ACDA                  45

MWC                   46

AAB                   47

GRP                   48

NFCR                  49

LNK                   51

FCA                   52

SR                    53

AA                    54

HIST                  55

AOP                   56

BARS                  57

NARS                  58

CDP                   59

PQUE                  60

FCBQ                  61

OHQ                   62

NAUT                  63

SNR                   64

NXFR                  67

HOT                   70

DHLD                  71

LSEL                  72

SS5                   73

DRNG                  74

PBXI                  75

DLDN                  76

CSL                   77

OOD                   79

SCI                   80

CCOS                  81

CDRQ                  83

TENS                  86

FTDS                  87

DSET                  88

TSET                  89

LNR                   90

DLT2                  91

PXLT                  92

SUPV                  93

CPND                  95

DNIS                  98

BGD                   99

RMS                   100

MR                    101

AWU                   102

PMSI                  103

LLC                   105

MCT                   107

ICDR                  108

APL                   109

TVS                   110

TOF                   111

IDC                   113

DCP                   115

PAGT                  116

CBC                   117

CCDR                  118

EMUS                  119

PLDN                  120

SCMP                  121

FTC                   125

BKI                   127

DTI2                  129

TBAR                  132

ENS                   133

FFC                   139

DCON                  140

MPO                   141

ISDN                  145

PRA                   146

ISL                   147

NTWK                  148

IEC                   149

DNXP                  150

CDRE                  151

FXS                   152

IAP3P                 153

PRI2                  154

THF                   157

FGD                   158

NAS                   159

FNP                   160

ISDN_INTL_SUP         161

SAR                   162

MINT                  163

LAPW                  164

GPRI                  167

ARIE                  170

CPGS                  172

ECCS                  173

AAA                   174

NMS                   175

EOVF                  178

HVS                   179

DKS                   180

SACP                  181

VNS                   183

OVLP                  184

EDRG                  185

POVR                  186

SECL                  191

ORC-RVQ               192

AINS                  200

IPRA                  202

XPE                   203

XCT0                  204

XCT1                  205

MLWU                  206

HSE                   208

MLM                   209

MAID                  210

VAWU                  212

EAR                   214

ECT                   215

BRI                   216

IVR                   218

MWI                   219

MSDL                  222

FC68                  223

SSAU                  229

BRIT                  233

FCDR                  234

BRIL                  235

MCMO                  240

MULTI_USER            242

ALRM_FILTER           243

SYS_MSG_LKUP          245

VMBA                  246

CALL_ID               247

DPNA                  250

SCDR                  251

ARFW                  253

PHTN                  254

ADMINSET              256

ATX                   258

CDRX                  259

QSIG                  263

NI-2                  291

IPEX                  295

MAT                   296

CPP                   301

QSIGGF                305

CPRKNET               306

PAGENET               307

CPCI                  310

TATO                  312

OPEN_ALARM            315

QSIG-SS               316

NGEN                  324

RANBRD                327

MUSBRD                328

ESA                   329

ESA_SUPP              330

ESA_CLMP              331

CNUMB                 332

CNAME                 333

NI-2_CBC              334

MEET                  348

MC32                  350

DBA                   351

FDID                  362

NMCE                  364

STS_MSG               380

CDIR                  381

VIRTUAL_OFFICE        382

ATAN                  384

NI2NAME               385

M3900_PROD_ENH        386

VIR_OFF_ENH           387

OAS                   394

ICON                  397

PCA                   398

H323_VTRK             399

LOCX                  400

PVQM                  401

SIP                   406

## Configuring the Nortel CS1000M:  Switch 2 PBX ‘B’

Note:  It is important to set EXTT parameter to “yes” under LD 15 to avoid dropped calls on certain conference scenarios when the initiating/conferencing party drops (e.g., Phone A calls Phone C, Phone A conferences in Phone D, and Phone A drops).

********************************************************************************

COMMON EQUIPMENT COMFIGURATION

********************************************************************************

>ld 22

PT2000

REQ  prt

TYPE cequ

CEQU

MPED 8D

SUPL  000  004  008  012

016  032  036  040

044  048  064  068

072 V096

TDS   000

CONF  029  030  031  062

094  095

DLOP  NUM DCH FRM TMDI LCMT YALM T1TE TRSH

PRI  06   23  ESF NO   B8S  FDL  -    00

MISP

********************************************************************************

DIGITAL DATA BLOCK CONFIGURATION

********************************************************************************

>ld 73

18

DDB000

MEM AVAIL: (U/P): 2757315    USED U P: 317254 38390    TOT: 3112959

SCH5066

REQ  prt

TYPE ddb

CC0  6

PREF CC0  6

SREF CC0  FREE RUN

TRSH 00

RALM 128

BIPC 0

LFAC 0

BIPV 4 4

SRTK 24 3600

SRNT 1024 1024

LFAL 10240 10240

SRIM 1

SRMM 2

ICS

********************************************************************************

D-CHANNEL CONFIGURATION

********************************************************************************

>ld 22

PT2000

REQ  prt

TYPE adan dch 6

ADAN     DCH 6

CTYP MSDL

CARD 06

PORT 1

DES  CS102

USR  PRI

DCHL 6

OTBF 32

PARM RS422  DTE

DRAT 64KC

CLOK EXT

IFC  ISGF

PINX_CUST 0

ISDN_MCNT 300

CLID OPT0

CO_TYPE  STD

SIDE USR

CNEG 1

RLS  ID  4

RCAP COLP NDI CCBI CCNI PRI DV3I CTI  QMWI

PR_TRIGS DIV 2 3

CNG 2 3

CON 2 3

CTR2 2 3

PR_RTN NO

MBGA NO

OVLR YES

DIDD 0

OVLS YES

OVLT 0

T310 120

T200 3

T203 10

N200 3

N201 260

K    7

********************************************************************************

ROUTE DATA BLOCK CONFIGURATION

********************************************************************************

>ld 21

PT1000

REQ: prt

TYPE: rdb

CUST 0

ROUT 106

TYPE RDB

CUST 00

DMOD

ROUT 106

DES

TKTP TIE

NPID_TBL_NUM   0

ESN  NO

CNVT NO

SAT  NO

RCLS EXT

VTRK NO

NODE

DTRK YES

BRIP NO

DGTP PRI

ISDN YES

MODE PRA

IFC  ISGF

SBN  NO

PNI  00001

NCNA NO

NCRD NO

CHTY BCH

CTYP UKWN

INAC NO

ISAR NO

CPFXS YES

DAPC NO

INTC NO

DSEL VOD

PTYP PRI

AUTO NO

DNIS NO

DCDR NO

ICOG IAO

SRCH RRB

TRMB YES

STEP

ACOD 506

TCPP NO

TARG 01

CLEN 1

BILN NO

OABS

INST

ANTK

SIGO STD

ICIS YES

TIMR ICF  512

OGF  512

EOD  13952

NRD  10112

DDL  70

ODT  4096

RGV  640

GRD  896

SFB  3

NBS  2048

NBL  4096

IENB  5

TFD  0

VSS  0

VGD  6

DRNG NO

CDR  NO

VRAT NO

MUS  NO

OHQ  NO

OHQT 00

CBQ  NO

AUTH NO

TTBL 0

ATAN NO

PLEV 2

ALRM NO

ART  0

SGRP 0

AACR NO

********************************************************************************

TRUNK DATA BLOCK CONFIGURATION

********************************************************************************

REQ: prt

TYPE: tnb

TN   6 1

DATE

PAGE

22

DES

22

DES

TN   006 01

TYPE TIE

CDEN SD

CUST 0

TRK  PRI

PDCA 1

PCML MU

NCOS 0

RTMB 106 1

B-CHANNEL SIGNALING

TGAR 1

AST  NO

IAPG 0

CLS  UNR DTN WTA LPR APN THFD HKD

P10 VNL

TKID

AACR NO

DATE 18 MAY 2005

********************************************************************************

COORDINATED DIALING PLAN CONFIGURATION (TO DIAL 23xx)

********************************************************************************

>ld 87

ESN000

MEM AVAIL: (U/P): 2757315    USED U P: 317254 38390    TOT: 3112959

SCH5066

REQ  prt

CUST 0

FEAT cdp

TYPE dsc

DSC  23

DSC  23

FLEN 0

DSP  LSC

RLI  6

NPA

NXX

********************************************************************************

ROUTE LIST BLOCK CONFIGURATION

********************************************************************************

OVL000

>ld 86

ESN000

MEM AVAIL: (U/P): 2757315    USED U P: 317254 38390    TOT: 3112959

SCH5066

REQ  prt

CUST 0

FEAT rlb

RLI  6

RLI  6

ENTR 0

LTER NO

ROUT 106

TOD  0 ON  1 ON  2 ON  3 ON

4 ON  5 ON  6 ON  7 ON

VNS  NO

CNV  NO

EXP  NO

FRL  0

DMI  0

FCI  0

FSNI 0

SBOC NRR

IDBB DBD

IOHQ NO

OHQ  NO

CBQ  NO

ISET 0

NALT 5

MFRL 0

OVLL 2

********************************************************************************

DIGITAL STATION PHONE CONFIGURATION

********************************************************************************

>ld 11

REQ: prt

TYPE: 2616

TN   001 0 0 0

DATE

PAGE

DES

DES  CS102

TN   001 0 00 00

TYPE 2616

CDEN 8D

CUST 0

AOM  0

FDN  2500

TGAR 1

LDN  NO

NCOS 0

SGRP 0

RNPG 0

SCI  0

SSU

XLST

CLS  CTD FBA WTA LPR MTD FNA HTA ADD HFD

MWA LMPN RMMD SMWD AAD IMD XHD IRD NID OLD VCE DRG1

POD DSX VMD CMSD SLKD CCSD SWD LND CNDA

CFTA SFD MRD DDV CNID CDCA MSID DAPA BFED RCBD

ICDD CDMD LLCN MCTD CLBD AUTU

GPUD DPUD DNDA CFXA ARHD CLTD ASCD

CPFA CPTA ABDD CFHD FICD NAID BUZZ AGRD MOAD AHD

DDGA NAMA

DRDD EXR0

USRD ULAD RTDD RBDD RBHD PGND FLXD FTTC DNDY DNO3 MCBN CDMR

CPND_LANG ENG

RCO  0

EFD  2500

HUNT 2500

EHT  2500

LHK  0

PLEV 02

CSDN

AST

IAPG 0

AACS NO

ITNA NO

DGRP

MLWU_LANG 0

DNDR 0

KEY  00 SCR 5308 0     MARP

CPND

NAME ATHENA8

XPLN 13

DISPLAY_FMT FIRST,LAST

01

02

03 CFW 4  2308

04 AO6

05 TRN

06

07

08

09

10

11

12

13

14

15 RGA

DATE  1 JUN 2005

********************************************************************************

PBX SOFTWARE RELEASE

********************************************************************************

>ld 22

26

PT2000

REQ  iss

26

CALL SERVER/MAIN CAB

VERSION 2121

RELEASE 4

ISSUE 00 T  +

IDLE_SET_DISPLAY NORTEL

********************************************************************************

PACKAGES INSTALLED

********************************************************************************

REQ  prt

TYPE pkg

OPTF                  1

CUST                  2

CDR                   4

CTY                   5

RAN                   7

TAD                   8

DNDI                  9

EES                   10

INTR                  11

ANI                   12

ANIR                  13

BRTE                  14

DNDG                  16

MSB                   17

SS25                  18

DDSP                  19

ODAS                  20

DI                    21

CHG                   23

CAB                   24

BAUT                  25

CASM                  26

CASR                  27

BQUE                  28

NTRF                  29

NCOS                  32

CPRK                  33

SSC                   34

IMS                   35

UST                   35

UMG                   35

ROA                   36

NSIG                  37

MCBQ                  38

NSC                   39

BACD                  40

ACDB                  41

ACDC                  42

LMAN                  43

MUS                   44

ACDA                  45

MWC                   46

AAB                   47

GRP                   48

NFCR                  49

LNK                   51

FCA                   52

SR                    53

AA                    54

HIST                  55

AOP                   56

BARS                  57

NARS                  58

CDP                   59

PQUE                  60

FCBQ                  61

OHQ                   62

NAUT                  63

SNR                   64

NXFR                  67

HOT                   70

DHLD                  71

LSEL                  72

SS5                   73

DRNG                  74

PBXI                  75

DLDN                  76

CSL                   77

OOD                   79

SCI                   80

CCOS                  81

CDRQ                  83

TENS                  86

FTDS                  87

DSET                  88

TSET                  89

LNR                   90

DLT2                  91

PXLT                  92

SUPV                  93

CPND                  95

DNIS                  98

BGD                   99

RMS                   100

MR                    101

AWU                   102

PMSI                  103

LLC                   105

MCT                   107

ICDR                  108

APL                   109

TVS                   110

TOF                   111

IDC                   113

DCP                   115

PAGT                  116

CBC                   117

CCDR                  118

EMUS                  119

PLDN                  120

SCMP                  121

FTC                   125

BKI                   127

DTI2                  129

TBAR                  132

ENS                   133

FFC                   139

DCON                  140

MPO                   141

ISDN                  145

PRA                   146

ISL                   147

NTWK                  148

IEC                   149

DNXP                  150

CDRE                  151

FXS                   152

IAP3P                 153

PRI2                  154

THF                   157

FGD                   158

NAS                   159

FNP                   160

ISDN_INTL_SUP         161

SAR                   162

MINT                  163

LAPW                  164

GPRI                  167

ARIE                  170

CPGS                  172

ECCS                  173

AAA                   174

NMS                   175

EOVF                  178

HVS                   179

DKS                   180

SACP                  181

VNS                   183

OVLP                  184

EDRG                  185

POVR                  186

SECL                  191

ORC-RVQ               192

AINS                  200

IPRA                  202

XPE                   203

XCT0                  204

XCT1                  205

MLWU                  206

HSE                   208

MLM                   209

MAID                  210

VAWU                  212

EAR                   214

ECT                   215

BRI                   216

IVR                   218

MWI                   219

MSDL                  222

FC68                  223

SSAU                  229

BRIT                  233

FCDR                  234

BRIL                  235

MCMO                  240

MULTI_USER            242

ALRM_FILTER           243

SYS_MSG_LKUP          245

VMBA                  246

CALL_ID               247

DPNA                  250

SCDR                  251

ARFW                  253

PHTN                  254

ADMINSET              256

ATX                   258

CDRX                  259

QSIG                  263

NI-2                  291

IPEX                  295

MAT                   296

CPP                   301

QSIGGF                305

CPRKNET               306

PAGENET               307

CPCI                  310

TATO                  312

OPEN_ALARM            315

QSIG-SS               316

NGEN                  324

RANBRD                327

MUSBRD                328

ESA                   329

ESA_SUPP              330

ESA_CLMP              331

CNUMB                 332

CNAME                 333

NI-2_CBC              334

MEET                  348

MC32                  350

DBA                   351

FDID                  362

NMCE                  364

STS_MSG               380

CDIR                  381

VIRTUAL_OFFICE        382

ATAN                  384

NI2NAME               385

M3900_PROD_ENH        386

VIR_OFF_ENH           387

OAS                   394

ICON                  397

PCA                   398

H323_VTRK             399

LOCX                  400

PVQM                  401

SIP                   406

## Configuring the Cisco 2801

2801_East#sho ver

Cisco IOS Software, 2801 Software (C2801-IPVOICE-M), Version 12.4(2.10), INTERIM

SOFTWARE

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2005 by Cisco Systems, Inc.

Compiled Mon 13-Jun-05 12:55 by hqluong

ROM: System Bootstrap, Version 12.3(8r)T6, RELEASE SOFTWARE (fc1)

ROM: Cisco IOS Software, 2801 Software (C2801-IPVOICE-M), Version 12.4(1.8)T, IN

TERIM SOFTWARE

2801_East uptime is 6 days, 43 minutes

System returned to ROM by reload at 23:43:23 UTC Thu Jun 30 2005

System image file is "flash:c2801-ipvoice-mz.124-2.10"

Cisco 2801 (revision 4.1) with 116736K/14336K bytes of memory.

Processor board ID FHK084410UA

2 FastEthernet interfaces

24 Serial interfaces

2 Channelized T1/PRI ports

4 DSPs, 64 Voice resources

DRAM configuration is 64 bits wide with parity disabled.

191K bytes of NVRAM.

62592K bytes of ATA CompactFlash (Read/Write)

Configuration register is 0x2102

2801_East#wr t

Building configuration...

Current configuration : 2034 bytes

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname 2801_East

!

boot-start-marker

boot system flash c2801-ipvoice-mz.124-2.10

boot-end-marker

!

logging buffered 51200 warnings

enable secret 5 $1$VFMp$KDnzbZkIOvIWZ0tnLHnK40

!

no aaa new-model

!

resource policy

!

network-clock-participate wic 2

mmi polling-interval 60

no mmi auto-configure

no mmi pvc

mmi snmp-timeout 180

ip subnet-zero

ip cef

!

!

!

!

ip domain name yourdomain.com

isdn switch-type primary-qsig

!

voice-card 0

!

!

!

voice service voip

signaling forward unconditional

sip

!

!

!

username deepa

!

!

controller T1 0/2/0

framing esf

linecode b8zs

pri-group timeslots 1-24

description ECN-5

!

controller T1 0/2/1

framing esf

linecode b8zs

!

!

!

interface FastEthernet0/0

description $ETH-LAN$$ETH-SW-LAUNCH$$INTF-INFO-FE 0$

ip address 172.20.4.111 255.255.255.0

duplex auto

speed auto

!

interface FastEthernet0/1

ip address 10.10.10.1 255.255.255.0

shutdown

duplex auto

speed auto

!

interface Serial0/2/0:23

description D-channel for ECN-5

no ip address

no logging event link-status

isdn switch-type primary-qsig

isdn overlap-receiving

isdn incoming-voice voice

isdn supp-service name calling

isdn bchan-number-order ascending

isdn sending-complete

no cdp enable

!

ip classless

!

ip http server

ip http authentication local

!

disable-eadi

!

!

!

control-plane

!

!

!

voice-port 0/2/0:23

description voice port for ECN-5

!

!

dial-peer voice 323 voip

destination-pattern 5...

session target ipv4:172.20.4.109

!

dial-peer voice 2023 pots

destination-pattern 2...

direct-inward-dial

port 0/2/0:23

forward-digits all

!

dial-peer voice 5050 voip

destination-pattern 5050

session target ipv4:172.20.4.109

!

gateway

timer receive-rtp 1200

!

sip-ua

!

!

line con 0

line aux 0

line vty 0 4

privilege level 15

no login

transport input telnet

line vty 5 15

privilege level 15

no login

transport input telnet

line vty 16 51

no login

!

end

2801_East#sho isdn stat

Global ISDN Switchtype = primary-qsig

ISDN Serial0/2/0:23 interface

dsl 0, interface ISDN Switchtype = primary-qsig

**** Slave side configuration ****

Layer 1 Status:

ACTIVE

Layer 2 Status:

TEI = 0, Ces = 1, SAPI = 0, State = MULTIPLE_FRAME_ESTABLISHED

Layer 3 Status:

0 Active Layer 3 Call(s)

Active dsl 0 CCBs = 0

The Free Channel Mask:  0x807FFFFF

Number of L2 Discards = 0, L2 Session ID = 35

Total Allocated ISDN CCBs = 0

2801_East#

## Configuring the Cisco 3845

3845_East#sho ver

Cisco IOS Software, 3800 Software (C3845-IPVOICE-M), Version 12.4(2.10), INTERIM

SOFTWARE

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2005 by Cisco Systems, Inc.

Compiled Mon 13-Jun-05 09:25 by hqluong

ROM: System Bootstrap, Version 12.3(11r)T1, RELEASE SOFTWARE (fc1)

3845_East uptime is 6 days, 8 minutes

System returned to ROM by reload at 00:19:57 UTC Fri Jul 1 2005

System image file is "flash:c3845-ipvoice-mz.124-2.10"

Cisco 3845 (revision 1.0) with 224256K/37888K bytes of memory.

Processor board ID FHK0847F13F

2 Gigabit Ethernet interfaces

24 Serial interfaces

1 Channelized T1/PRI port

2 Voice FXS interfaces

DRAM configuration is 64 bits wide with parity enabled.

479K bytes of NVRAM.

62592K bytes of ATA System CompactFlash (Read/Write)

Configuration register is 0x2102

3845_East#wr t

Building configuration...

Current configuration : 2041 bytes

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname 3845_East

!

boot-start-marker

boot system flash:c3845-ipvoice-mz.124-2.10

boot-end-marker

!

logging buffered 51200 warnings

enable secret 5 $1$s2pF$7EEyO5PpjtSJxm4Rfy2wQ1

!

no aaa new-model

!

resource policy

!

no network-clock-participate slot 1

ip subnet-zero

ip cef

!

!

!

!

ip domain name yourdomain.com

isdn switch-type primary-qsig

voice-card 0

dspfarm

!

voice-card 1

dspfarm

!

!

!

username deepa

!

!

controller T1 1/0/0

framing esf

linecode b8zs

cablelength short 133

pri-group timeslots 1-24

description ECN-11

!

!

!

interface GigabitEthernet0/0

description $ETH-LAN$$ETH-SW-LAUNCH$$INTF-INFO-GE 0/0$

ip address 10.10.10.1 255.255.255.248

shutdown

duplex auto

speed auto

media-type rj45

negotiation auto

!

interface GigabitEthernet0/1

ip address 172.20.4.109 255.255.255.0

duplex auto

speed auto

media-type rj45

negotiation auto

!

interface Serial1/0/0:23

description D-channel for ECN-11

no ip address

no logging event link-status

isdn switch-type primary-qsig

isdn overlap-receiving

isdn protocol-emulate network

isdn incoming-voice voice

isdn supp-service name calling

isdn T310 120000

isdn sending-complete

no cdp enable

!

ip classless

!

ip http server

ip http authentication local

!

!

!

!

control-plane

!

!

!

voice-port 0/0/0

!

voice-port 0/0/1

!

voice-port 1/0/0:23

description voice port for ECN-11

!

!

!

!

!

dial-peer voice 323 voip

destination-pattern 2...

session target ipv4:172.20.4.111

!

dial-peer voice 10023 pots

destination-pattern 5...

direct-inward-dial

port 1/0/0:23

forward-digits all

!

dial-peer voice 5050 pots

destination-pattern 5050

direct-inward-dial

port 1/0/0:23

forward-digits all

!

!

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

privilege level 15

login local

transport input telnet

line vty 5 15

privilege level 15

login local

transport input telnet

!

scheduler allocate 20000 1000

!

end

3845_East#

3845_East#

3845_East#sho isdn stat

Global ISDN Switchtype = primary-qsig

ISDN Serial1/0/0:23 interface

******* Network side configuration *******

dsl 0, interface ISDN Switchtype = primary-qsig

**** Master side configuration ****

Layer 1 Status:

ACTIVE

Layer 2 Status:

TEI = 0, Ces = 1, SAPI = 0, State = MULTIPLE_FRAME_ESTABLISHED

Layer 3 Status:

0 Active Layer 3 Call(s)

Active dsl 0 CCBs = 0

The Free Channel Mask:  0x807FFFFF

Number of L2 Discards = 0, L2 Session ID = 106

Total Allocated ISDN CCBs = 0

3845_East#

# Acronyms

| Acronym | Definitions |
| --- | --- |

Important Information

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.
IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

| Corporate Headquarters Cisco Systems, Inc. 170 West Tasman Drive San Jose, CA 95134-1706 USA www.cisco.com Tel: 408 526-4000 800 553-NETS (6387) Fax: 408 526-4100 | European Headquarters Cisco Systems International BV Haarlerbergpark Haarlerbergweg 13-19 1101 CH Amsterdam The Netherlands www-europe.cisco.com Tel: 31 0 20 357 1000 Fax: 31 0 20 357 1100 | Americas Headquarters Cisco Systems, Inc. 170 West Tasman Drive San Jose, CA 95134-1706 USA www.cisco.com Tel: 408 526-7660 Fax: 408 527-0883 | Asia Pacific Headquarters Cisco Systems, Inc. Capital Tower 168 Robinson Road #22-01 to #29-01 Singapore 068912 www.cisco.com Tel: +65 317 7777 Fax: +65 317 7799 |
| --- | --- | --- | --- |
| Cisco Systems has more than 200 offices in the following countries and regions. Addresses, phone numbers, and fax numbers are listed on the Cisco Web site at www.cisco.com/go/offices. Argentina • Australia • Austria • Belgium • Brazil • Bulgaria • Canada • Chile • China PRC • Colombia • Costa Rica • Croatia • Czech Republic • Denmark • Dubai, UAE • Finland • France • Germany • Greece • Hong Kong SAR • Hungary • India • Indonesia • Ireland • Israel • Italy • Japan • Korea • Luxembourg • Malaysia • Mexico • The Netherlands • New Zealand • Norway • Peru • Philippines • Poland • Portugal • Puerto Rico • Romania • Russia • Saudi Arabia • Scotland • Singapore • Slovakia • Slovenia • South Africa • Spain • Sweden • Switzerland • Taiwan • Thailand • Turkey Ukraine • United Kingdom • United States • Venezuela • Vietnam • Zimbabwe © 2007 Cisco Systems, Inc. All rights reserved. CCVP, the Cisco logo, and Welcome to the Human Network are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn is a service mark of Cisco Systems, Inc.; and Access Registrar, Aironet, Catalyst, CCDA, CCDP, CCIE, CCIP, CCNA, CCNP, CCSP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Cisco Unity, Enterprise/Solver, EtherChannel, EtherFast, EtherSwitch, Fast Step, Follow Me Browsing, FormShare, GigaDrive, HomeLink, Internet Quotient, IOS, iPhone, IP/TV, iQ Expertise, the iQ logo, iQ Net Readiness Scorecard, iQuick Study, LightStream, Linksys, MeetingPlace, MGX, Networkers, Networking Academy, Network Registrar, PIX, ProConnect, ScriptShare, SMARTnet, StackWise, The Fastest Way to Increase Your Internet Quotient, and TransPath are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the United States and certain other countries. All other trademarks mentioned in this document or Website are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0711R) Printed in the USA |  |  |  |